CS_03550-1D Software:

Revision History: 1C

Test Program revision changes. Added HP VEE Test Data Server Setup.

Revision History: 1D

Added automated VSWR, Test Port Insertion Loss and Test Port Isolation tests.

CS_03550-1 Software supports testing for N12-3541 and N15-5350 (Semi-Automated Testing)

Revision B added a Test Data Server capability. Revision B software requires that a server directory be setup on one of the computers on the network. All computers running the revision B software must map the server file as their A:\ drive. Details are described below in the Server Section.

Revision C added Phase Testing capability. This test is specifically used for automating phase measurements on the N12-3541 and N15-5350 products.

Revision D added VSWR, Test Port Insertion Loss and Test Port Isolation semi-automated testing capability. This captures measurements from the scalar analyzer and saves it to the data record for Pre-Test and Final-Test data sheets.

Test Capability includes: Gain Testing, Temperature Sensor Testing, Noise Figure Testing, Input P1dB Compression Point Testing, Mixer Spur Testing, Phase Testing, VSWR, Test Port Insertion Loss and Test Port Isolation.

Each of the above tests are accomplished with independent test programs as follows:

HP VEE		N12-3541 Gain Test

Arduino		N12-3541 Temperature Sensor Test

NI LabView	N12-3541 Noise Figure Test

HP VEE		N12-3541 P1dB Test

HP VEE		N12-3541 Mixer Spurs Test

Excel VBA	N12-3541 Phase Test

HP VEE		N12-3541 VSWR Test

Test Equipment as listed, or equivalent may be used.

HP VEE TEST DATA SERVER SETUP

Test Program Folder:	 VEE_SERVER_A1

To Setup Server

Install One Directory under the C: Drive (On the server computer)

VEE_SERVER

(Test data files are stored in subdirectories in this location.)

Ensure all computers writing data to this server have mapped their A:\ drive to the VEE_SERVER  Directory Location.

The setup for each test program is unique and is documented below:

HP VEE		N12-3541 Gain Test Setup

Test Program Folder:	 N12-3541 Gain_B1

To Setup and Run Test Program (N12-3541 Gain_B1.vxe – In VEE Programs Directory)

Install Three Directories under the C: Drive

VEE Programs; VEE Setup Data; VEE Test Data (Test data files are stored in this location.)

Install Agilent VEE Library 17.2; Agilent VEE 9.32; and Microsoft EXCEL (Office 12 or later)

Set test equipment to the correct HPIB addresses – As shown below

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)	GPIB Address:

HP8757D		Scalar Analyzer						16

HP83640A		RF Source (10MHz to 40 GHz)				19

Desktop/Laptop with Windows 10 OS

HP 82357B USB/GPIB Interface

Arduino		N12-3541 Temperature Sensor Test

Test Program Folder:	 N12-3541 Temperature Sensor Test_A1

To Setup and Run Test Program (Arduino.exe)

Install Arduino 1.8.5

Install Test Program file (Delay_SPI_Temp_Sensor_Multi_Command_2012_08_14.ino) in directory Documents\Arduino\Delay_SPI_Temp_Sensor_Multi_Command_2012_08_14

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)

Desktop/Laptop with Windows 10 OS

NI LabView		N12-3541 Noise Figure Test Setup

Test Program Folder:	 N12-3541 Noise Figure_B1

To Setup and Run Test Program (N12-3541 Noise Figure_B1.exe)

Install One Directory under the C: Drive

AUTOTEST (AUTOTEST has three sub-directories (CALIBRATION, NI TEST DATA, and REF) and the application File (N12-3541 Noise Figure_B1.exe)

Install LABVIEW 2018 (LVRTE2018SP1_f3Patchstd.exe) – for 32 BIT Applications

Set test equipment to the correct HPIB addresses – As shown below

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)	GPIB Address:

HP8757D		Noise Figure Meter					Anything

HP83640A		RF Source (10MHz to 40 GHz)				Anything

Desktop/Laptop with Windows 10 OS

HP 82357B USB/GPIB Interface

HP VEE		N12-3541 P1 dB Test Setup

Test Program Folder:	 N12-3541 P1 dB_B1

To Setup and Run Test Program (N12-3541 P1_dB_B1.vxe – In VEE Programs Directory)

Install Four Directories under the C: Drive

VEE Programs; VEE Calibration Data; VEE Setup Data; VEE Test Data

Install Agilent VEE Library 17.2; Agilent VEE 9.32; and Microsoft EXCEL (Office 12 or later)

Set test equipment to the correct HPIB addresses – As shown below

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)	GPIB Address:

HP83650B		RF Source (10MHz to 50 GHz)				19

HP438A			Power Meter						13

Desktop/Laptop with Windows 10 OS

HP 82357B USB/GPIB Interface

HP VEE		N12-3541 Mixer Spurs Test Setup

Test Program Folder:	 N12-3541 Mixer Spurs_B1

To Setup and Run Test Program (N12-3541 Mixer Spurs_B1.vxe – In VEE Programs Directory)

Install Four Directories under the C: Drive

VEE Programs; VEE Calibration Data; VEE Setup Data; VEE Test Data

Install Agilent VEE Library 17.2; Agilent VEE 9.32; and Microsoft EXCEL (Office 12 or later)

Set test equipment to the correct HPIB addresses – As shown below

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)	GPIB Address:

HP83650B		RF Source (10MHz to 50 GHz)				19

HP438A			Power Meter						13

HP8565E		Spectrum Analyzer (10MHz to 50 GHz)			14

Desktop/Laptop with Windows 10 OS

HP 82357B USB/GPIB Interface

Excel VBA		N12-3541 Phase Test Setup

Test Program Folder:	 N12-3541 Phase_A1

To Setup and Run Test Program (Block_DC_Final_Test.xltm – In VEE Programs Directory)

Please refer to CS-03550-1C Appendix A (separate file) for installation notes.

Install Two Directories under the C: Drive

VEE Programs; VEE Test Data (Test data files are stored in this location.)

Install Agilent VEE Library 17.2; Agilent VEE 9.32; and Microsoft EXCEL (Office 12 or later)

Set test equipment to the correct HPIB addresses – As shown below

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)	GPIB Address:

HP8720B		Vector Analyzer						16

Desktop/Laptop with Windows 10 OS

HP 82357B USB/GPIB Interface

HP VEE		N12-3541 VSWR and Test Port Insertion & Isolation; Test Setups

Test Program Folder:	 N12-3541 VSWR

To Setup and Run Test Program (N12-3541 VSWR.vxe – In VEE Programs Directory)

Install three Directories under the C: Drive

VEE Programs; VEE Setup Data; VEE Test Data (Test data files are stored in this location.)

Under VEE Setup Data place the following 3 pictures called by this program

N12-3541 IF J7 VSWR Test SetUp.jpg

N12-3541 J6 Test Port VSWR, Insertion & Switch Isolation Test SetUp.jpg

N12-3541 RF VSWR Test SetUp.jpg

Also in this directory insure that the data template is the most current version

N12-3541 RF VSWR Test SetUp.xlsx

N15-5350 RF VSWR Test SetUp.xlsx

Install the same three Directories under the A: Drive (also known as the Server)

VEE Programs; VEE Setup Data; VEE Test Data (Test data files are stored in this location.)

Under the Server VEE Setup Data ensure the current data templates are present.

N12_3541_A1_Final_Data_Sheet.xlsx

N15_5350_A1_Final_Data_Sheet.xlsx

Install Agilent VEE Library 17.2; Agilent VEE 9.32; and Microsoft EXCEL (Office 12 or later)

Please refer to ATP-03550-1D Appendix A section 6.7 for the VSWR test usage instructions.

Set test equipment to the correct HPIB addresses – As shown below

Test Equipment Required: (Reference ATP-03550 for additional Equipment Required.)	GPIB Address:

HP8757D		Scalar Analyzer						16

HP83640A		RF Source (10MHz to 40 GHz)				19

Desktop/Laptop 		Windows 10 OS

HP 82357B 		USB/GPIB Interface

HP 85027D		Directional Bridge

Gain Test Setup:

Temperature Sensor Test Setup:

Test Port

NC J6

UUT

NC J5

NC J4

DC Power Supplies

& Controls

Figure 6.2.  Temperature Sensor Test Set-Up

Computer

NC J3

NC J2

NC J7

IF

P1

NC J1

- 6.0 Vdc

+8.5 Vdc

+5.0 Vdc

Noise Figure Test Setup:

P1 dB and Mixer Spurs Calibration Setup:

P1 dB Test Setup:

Mixer Spurs Test Setup (N12-3541):

Mixer Spurs Test Setup (N12-3542):

Phase Tracking Test Setup (N12-3541):

Signal Upconverter

Signal Generator

Vector Network Analyzer

11 GHz Signal Source

DC Power Supply & Controls

Figure 6.4.1. Phase Tracking Test Set-Up

Power Divider

J3

R

I

L

-10 dB

UUT

R

L

I

- 6.0 Vdc

+8.5 Vdc

+5.0 Vdc

10 MHz Ref

X4

X4

J5

J7

J1-J4

J6

N12-3542

J7

J4

Port 1

Port 2

P1

DC Power Supply & Controls

- 6.0 Vdc

+8.5 Vdc

P1

Use Gold Standard Unit for Calibration

VSWR: IF, RF & Test Port; and Test Port Insertion Loss & Test Port Switch On/Off Isolation: