Appendix B,  CS_03550-1C

Usage notes for VSWR test:

- Saved registers

The VSWR program expects registers 2 , 3, 5 to be calibrated and validated prior to running.  The LMCO benches have a standard register assignment. See below.

Recall Registers  HP8757D

1. down converter gain

2. J6, J7 insertion loss & VSWR

3. down converter J1-J4 VSWR

4. Up Converter gain & VSWR

5. J6 to J7 isolation

6. up converter roll off

7. input stopband

Only registers 1 thru 4 store calibration memory, along with the instrument state. The register 5, J6-J7 isolation, is calibrated per unit: the tru is calibrated with cntrl 16, and isolation is measured with cntrl 17.

- Final Data Sheet

The Final Data workbook has added 2 worksheets , “decoder” and “VSWR”. The worksheet VSWR contains all the VSWR data, analysis, and graphs for efficient final test flow.  The worksheet “decoder” contains the indirect name for the Final Data Sheet to find VSWR data.

- Refer to the QUALIFICATION DESIGN VERIFICATION TEST /ACCEPTANCE TEST PROCEDURE for VSWR calibration settings.